Nomenclature:

CAS: Crank Angle Synchronous

AI: Analog Input

DI: Digital Input

CAD: Crank Angle Degree

| SCADA project DATA LOGGING |  |
| --- | --- |
| Topic: DATA LOGGING Author: C Guardiola, UPV Date: 31/01/2024 | Reviewer: Date: |
| OBJECTIVES: - To provide a factual view of the system operation, including measurements, calculated values, and actuators. - To log system operation. - To accelerate the postprocessing and test analysis. - To allow the postmortem analysis in case of errors triggering forced mode transitions (e.g. safety stop and venting) [TO BE VERIFIED] |  |
| GENERAL STRUCTURE: Figure 1 presents the general data flow structure, from acquisition to the stored files in the SSD disks connected to the two CRIO chassis. Whenever possible, all variables are propagated to the different files. Four files are generated: A.1 Crank angle synchronous (CAS) instantaneous AI: in-cycle solved value of the fast CAS AI tasks. A.2 Cycle averaged values, including relevant cylinder pressure and heat release calculation parameters. This file also includes the value of all time synchronous (TS) variables for every cycle. A.3 Test averaged values (average, standard deviation, minimum and maximum values of the previous file). B Time synchronous data logging. Figure 1. General data flow structure for the data logging. Red: variables from membrane controller; blue: time synchronous variables from CRIO905; green: crank angle synchronous variables from CRIO9049. |  |
| FILES and VARIABLE NAMING: - CAS FILES A.1. CAS instantaneous data file: - Format: TDMS (.tdms). 1 file per test. - Triggered by the test operator or by errors. - In_cycle evolution of the signals, according to encoder resolution (e.g. 0.1 CAD/sample) - Raw values (after sensor calibration) of the AI channels in the CAS mode - Up to 1000 cycles. User can select a lower cycle number. 7200 rows per cycle (with 0.1 CAD/sample). Error triggered or pre_trigger recording mode always stored all data in buffer (max 1000 cycles). - Automatic variable naming, pulled from configuration files. - Automated file naming: APC_CRIO9049_DATE_HOUR_ TESTCODE_ID_CAS.tdms (e.g. APC_CRIO9049_231010_103306_CO2swept_0013_CAS.tdms) - Included signals: all CAS AI inputs (NI9222 cards): SIGNALS in CAS AI; SIGNAL DEFINITION. Pcyl1_bar; E.g. Pressure within the main chamber of Cylinder #1, where the numbering of cylinders follows the convention outlined in the referenced document "Ref. Other document".. Pcyl2_bar. Pcyl3_bar. Pcyl4_bar. Pcyl5_bar. Pcyl6_bar. Ppre_bar. Psyst_bar. Pexh_bar. Note: 8 channels * 7200 floats * 1000 cycles >450 MB in TDMS (binary) format. A.2. Cycle averaged file: - Format: TDMS (.tdms). 1 file per test. - Triggered by the test operator or by errors. - Cycle averages of relevant variables: cylinder pressure metrics, combustion metrics, etc., plus value of the variables streamed from the time synchronous tasks (control settings, AI, etc.) - Up to 1000 cycles. User can select a lower cycle number (see A.1). 1 row per cycle. - Automatic naming extracted from a configuration files. - File naming: APC_CRIO9049_DATE_HOUR_TESTCODE_ID_ CycleAVG.tdms (e.g. APC_CRIO9049_231010_103306_CO2swept_0013_CycleAVG.tdms) - Included signals: CRIO9049 controls; CAS calculation results; CRIO9056 variables. TimeStamp9049_ms; WF_PT_012avg_bar; TimeStamp9056_ms. TimeStamp9056echo_ms; WF_PT_005avg_bar; SystemState_i8. InjectionState_b; IMEPg1_bar; WF_PT_001_bar. InjEnable_b; IMEPg2_bar; WF_OA_001_O2_perc. MainEnable_b; IMEPg3_bar; WF_OA_001_O2corr_perc. MainDuration_ms; IMEPg4_bar; WF_OA_001_Lambda_r. MainSOI_CADBTDC; IMEPg5_bar; WF_OA_002_O2_perc. SparkEnable_b; IMEPg6_bar; WF_OA_002_Lambda_r. SparkTiming_CADBTDC; IMEPn1_bar; WF_OA_002_NOx_ppm. IMEPn2_bar; EO_PT_001_bar. IMEPn3_bar; TORQUE_Nm. IMEPn4_bar; NI9205_9_V (channels not used). IMEPn5_bar; NI9205_10_V. IMEPn6_bar; NI9205_11_V. PMEP1_bar; NI9205_12_V. PMEP2_bar; NI9205_13_V. PMEP3_bar; NI9205_14_V. PMEP4_bar; NI9205_15_V. PMEP5_bar; NG_FT_001_kgs. PMEP6_bar; NG_PT_001_bar. MaxPcyl1_bar; NGDI_PT_001_bar. MaxPcyl2_bar; O2_FC_001_kgs. MaxPcyl3_bar; NG_FC_001_kgs. MaxPcyl4_bar; NGDI_FT_001_kgs. MaxPcyl5_bar; WF_PT_004_bar. MaxPcyl6_bar; O2_PT_001_bar. IMEPstd1_bar; AR_PT_001_bar. IMEPstd2_bar; AR_FT_001_kgs. IMEPstd3_bar; WF_PT_016_bar. IMEPstd4_bar; WF_PT_014_bar. IMEPstd5_bar; SW_FT_001_kgs. IMEPstd6_bar; SW_FT_002_kgs. MAPO1_bar; SW_FT_003_kgs. MAPO2_bar; SW_FT_004_kgs. MAPO3_bar; WF_TT_001_degC. MAPO4_bar; O2_TT_001_degC. MAPO5_bar; AR_TT_001_degC. MAPO6_bar; NG_TT_001_degC. RI1 (placeholder for ringing intensity); NGDI_TT_001_degC. RI2; WF_TT_002 _degC. RI3; WF_TT_003_degC. RI4; WF_TT_004_degC. RI5; WF_TT_007_degC. RI6; PC_TT_001_degC. maxQ1_J; PC_TT_002_degC. maxQ2_J; PC_TT_003_degC. maxQ3_J; PC_TT_004_degC. maxQ4_J; PC_TT_005_degC. maxQ5_J; PC_TT_006_degC. maxQ6_J; EC_TT_001_degC. CA03_1_CADATDC; EC_TT_002_degC. CA03_2_CADATDC; SW_TT_001_degC. CA03_3_CADATDC; SW_TT_002_degC. CA03_4_CADATDC; SW_TT_003_degC. CA03_5_CADATDC; SW_TT_004_degC. CA03_6_CADATDC; SW_TT_005_degC. CA10_1_CADATDC; SW_TT_006_degC. CA10_2_CADATDC; SW_TT_007_degC. CA10_3_CADATDC; SW_TT_008_degC. CA10_4_CADATDC; SW_TT_009_degC. CA10_5_CADATDC; EO_TT_001_degC. CA10_6_CADATDC; EO_TT_002_degC. CA50_1_CADATDC; NI9214b_13_degC. CA50_2_CADATDC; NI9214b_14_degC. CA50_3_CADATDC; NI9214b_15_degC. CA50_4_CADATDC; NI9214b_16_degC. CA50_5_CADATDC; NI9264_AO0_V. CA50_6_CADATDC; NI9264_AO1_V. CA90_1_CADATDC; NI9264_AO2_V. CA90_2_CADATDC; NI9264_AO3_V. CA90_3_CADATDC; NI9264_AO4_V. CA90_4_CADATDC; NI9264_AO5_V. CA90_5_CADATDC; NI9264_AO6_V. CA90_6_CADATDC; NI9264_AO7_V. CA97_1_CADATDC; NI9264_AO8_V. CA97_2_CADATDC; NI9264_AO9_V. CA97_3_CADATDC; NI9264_AO10_V. CA97_4_CADATDC; NI9264_AO11_V. CA97_5_CADATDC; NI9264_AO12_V. CA97_6_CADATDC; NI9264_AO13_V. CA10CA90_1_CAD; NI9264_AO14_V. CA10CA90_2_CAD; NI9264_AO15_V. CA10CA90_3_CAD; O2_FC_001_REF_mA. CA10CA90_4_CAD; AR_PC_002_REF_mA. CA10CA90_5_CAD; NG_FC_001_REF_mA. CA10CA90_6_CAD; EC_FC_001_mA. maxdpdcad1_barCAD; SW_FC_004_REF_mA. maxdpdcad2_barCAD; SW_FC_009_mA. maxdpdcad3_barCAD; DYNO_REF_mA. maxdpdcad4_barCAD; NI9266_AO7_mA. maxdpdcad5_barCAD; NG_FV_001_b. maxdpdcad6_barCAD; O2_PV_004_b. kappa_comp1_r; AR_PV_004_b. kappa_comp2_r; WF_FC_001_b. kappa_comp3_r; WF_FC_002_b. kappa_comp4_r; WF_FC_003_b. kappa_comp5_r; NI9375_AO6_b. kappa_comp6_r; NI9375_AO7_b. kappa_exp1_r; NI9375_AO8_b. kappa_exp2_r; NI9375_AO9_b. kappa_exp3_r; NI9375_AO10_b. kappa_exp4_r; NI9375_AO11_b. kappa_exp5_r; NI9375_AO12_b. kappa_exp6_r; NI9375_AO13_b. CylPresWarnings_u64; NI9375_AO14_b. CylPresErrors_u64; NI9375_AO15_b. NI9375_AI0_b. NI9375_AI1_b. NI9375_AI2_b. NI9375_AI3_b. NI9375_AI4_b. NI9375_AI5_b. NI9375_AI6_b. NI9375_AI7_b. NI9375_AI8_b. NI9375_AI9_b. NI9375_AI10_b. NI9375_AI11_b. NI9375_AI12_b. NI9375_AI13_b. NI9375_AI14_b. NI9375_AI15_b. AIC103_CO2_ConcTarget_ack. AT103_CO2Conc. PV103_PressTarget_kPa_ack. PT103_Pressure_kPa. PV103_VlvPosTarget_pc_ack. PV103_VlvPos_pc_fb. AIC201_CO2_ConcTarget_ack. AT201_CO2Conc. FV301_FlowTarget_kgh_ack. FT301_Flow_kgh. FV301_VlvPosTarget_pc_ack. FV301_VlvPos_pc_fb. PV300_PressTarget_kPa_ack. PT300_Pressure_kPa. PV300_VlvPosTarget_pc_ack. PV300_VlvPos_pc_fb. C101_CompSpeed_rpm. PT104_Pressure_kPa. PT106_Pressure_kPa. PLC_Manual_Control. MTR_State. MTR_HeartBeat. PV103_Ctrl_Mode_act. FV301_Ctrl_Mode_act. PV300_Ctrl_Mode_act. MTR_Diagnostic_Status. A.3. Test averaged file: - Format: ASCII (.txt). 1 file for testing campaign/day. Each recording generates 4 rows in the file. - Triggered by the test operator or by errors. - AVG, STD, MIN, MAX of the cycle values for the test. - The results are added as 4 rows of a growing ASCII files. The following fields are created: - First columns of the file are: ID: corresponds to the ID in CAS and CycleAVG files Warning: 0 if the measurement is requested by the test operator. Error code if it is a file triggered by a system error. Method: AVG, STD. MIN or MAX (note: a data filter in Excel serves for filtering the rows accordingly). - Next, the columns equivalent to those of the CycleAVG file are added (same naming convention). - Finally, three additional columns are added: CAS file: name of related CAS file. CycleAVG file: name of related CycleAVG file. TEST NOTES: test notes added from the HMI by the operator. - File naming: APC_CRIO9049_DATE_TESTCODE.txt (e.g. APC_CRIO9049_231010_CO2swept.txt) A new file is created if the test code is changed (e.g. different test campaign). - Included signals: Identification; Signals; Files and comments. ID; *ALL from CycleAVG file*; CA_ file. Warning; CycleAVG_file. Method; TEST_NOTES. - TS FILE - Format: TDMS (.tdms). Automated streaming to disk. 1 file created on CRIO9056 startup. - 10 Hz streaming of measured and calculated TS signals, together with the averaged CAS signals polled from the shared variables. - File naming: APC_CRIO9056_DATE_HOUR_10Hz.txt A single file is created on control system startup. - Included signals: CRIO9056 variables; Signals retrieved from MTR PLC; Signals retrieved from CRIO9049. TimeStamp9056_ms; AIC103_CO2_ConcTarget_ack; TimeStamp9049_ms. SystemState_i8; AT103_CO2Conc; TimeStamp9056echo_ms. WF_PT_001_bar; PV103_PressTarget_kPa_ack; InjectionState_b. WF_OA_001_O2_perc; PT103_Pressure_kPa; InjEnable_b. WF_OA_001_O2corr_perc; PV103_VlvPosTarget_pc_ack; MainEnable_b. WF_OA_001_Lambda_r; PV103_VlvPos_pc_fb; MainDuration_ms. WF_OA_002_O2_perc; AIC201_CO2_ConcTarget_ack; MainSOI_CADBTDC. WF_OA_002_Lambda_r; AT201_CO2Conc; SparkEnable_b. WF_OA_002_NOx_ppm; FV301_FlowTarget_kgh_ack; SparkTiming_CADBTDC. EO_PT_001_bar; FT301_Flow_kgh; EngineSpeed_rpm. TORQUE_Nm; FV301_VlvPosTarget_pc_ack; WF_PT_012avg_bar. NI9205_9_V; FV301_VlvPos_pc_fb; WF_PT_005avg_bar. NI9205_10_V; PV300_PressTarget_kPa_ack; IMEPg1_bar. NI9205_11_V; PT300_Pressure_kPa; IMEPg2_bar. NI9205_12_V; PV300_VlvPosTarget_pc_ack; IMEPg3_bar. NI9205_13_V; PV300_VlvPos_pc_fb; IMEPg4_bar. NI9205_14_V; C101_CompSpeed_rpm; IMEPg5_bar. NI9205_15_V; PT104_Pressure_kPa; IMEPg6_bar. NG_FT_001_kgs; PT106_Pressure_kPa; IMEPn1_bar. NG_PT_001_bar; PLC_Manual_Control; IMEPn2_bar. NGDI_PT_001_bar; MTR_State; IMEPn3_bar. O2_FC_001_kgs; MTR_HeartBeat; IMEPn4_bar. NG_FC_001_kgs; PV103_Ctrl_Mode_act; IMEPn5_bar. NGDI_FT_001_kgs; FV301_Ctrl_Mode_act; IMEPn6_bar. WF_PT_004_bar; PV300_Ctrl_Mode_act; PMEP1_bar. O2_PT_001_bar; MTR_Diagnostic_Status; PMEP2_bar. AR_PT_001_bar; PMEP3_bar. AR_FT_001_kgs; PMEP4_bar. WF_PT_016_bar; PMEP5_bar. WF_PT_014_bar; PMEP6_bar. SW_FT_001_kgs; MaxPcyl1_bar. SW_FT_002_kgs; MaxPcyl2_bar. SW_FT_003_kgs; MaxPcyl3_bar. SW_FT_004_kgs; MaxPcyl4_bar. WF_TT_001_degC; MaxPcyl5_bar. O2_TT_001_degC; MaxPcyl6_bar. AR_TT_001_degC; IMEPstd1_bar. NG_TT_001_degC; IMEPstd2_bar. NGDI_TT_001_degC; IMEPstd3_bar. WF_TT_002 _degC; IMEPstd4_bar. WF_TT_003_degC; IMEPstd5_bar. WF_TT_004_degC; IMEPstd6_bar. WF_TT_007_degC; MAPO1_bar. PC_TT_001_degC; MAPO2_bar. PC_TT_002_degC; MAPO3_bar. PC_TT_003_degC; MAPO4_bar. PC_TT_004_degC; MAPO5_bar. PC_TT_005_degC; MAPO6_bar. PC_TT_006_degC; RI1. EC_TT_001_degC; RI2. EC_TT_002_degC; RI3. SW_TT_001_degC; RI4. SW_TT_002_degC; RI5. SW_TT_003_degC; RI6. SW_TT_004_degC; maxQ1_J. SW_TT_005_degC; maxQ2_J. SW_TT_006_degC; maxQ3_J. SW_TT_007_degC; maxQ4_J. SW_TT_008_degC; maxQ5_J. SW_TT_009_degC; maxQ6_J. EO_TT_001_degC; CA03_1_CADATDC. EO_TT_002_degC; CA03_2_CADATDC. NI9214b_13_degC; CA03_3_CADATDC. NI9214b_14_degC; CA03_4_CADATDC. NI9214b_15_degC; CA03_5_CADATDC. NI9214b_16_degC; CA03_6_CADATDC. NI9264_AO0_V; CA10_1_CADATDC. NI9264_AO1_V; CA10_2_CADATDC. NI9264_AO2_V; CA10_3_CADATDC. NI9264_AO3_V; CA10_4_CADATDC. NI9264_AO4_V; CA10_5_CADATDC. NI9264_AO5_V; CA10_6_CADATDC. NI9264_AO6_V; CA50_1_CADATDC. NI9264_AO7_V; CA50_2_CADATDC. NI9264_AO8_V; CA50_3_CADATDC. NI9264_AO9_V; CA50_4_CADATDC. NI9264_AO10_V; CA50_5_CADATDC. NI9264_AO11_V; CA50_6_CADATDC. NI9264_AO12_V; CA90_1_CADATDC. NI9264_AO13_V; CA90_2_CADATDC. NI9264_AO14_V; CA90_3_CADATDC. NI9264_AO15_V; CA90_4_CADATDC. O2_FC_001_REF_mA; CA90_5_CADATDC. AR_PC_002_REF_mA; CA90_6_CADATDC. NG_FC_001_REF_mA; CA97_1_CADATDC. EC_FC_001_mA; CA97_2_CADATDC. SW_FC_004_REF_mA; CA97_3_CADATDC. SW_FC_009_mA; CA97_4_CADATDC. DYNO_REF_mA; CA97_5_CADATDC. NI9266_AO7_mA; CA97_6_CADATDC. NG_FV_001_b; CA10CA90_1_CAD. O2_PV_004_b; CA10CA90_2_CAD. AR_PV_004_b; CA10CA90_3_CAD. WF_FC_001_b; CA10CA90_4_CAD. WF_FC_002_b; CA10CA90_5_CAD. WF_FC_003_b; CA10CA90_6_CAD. NI9375_AO6_b; maxdpdcad1_barCAD. NI9375_AO7_b; maxdpdcad2_barCAD. NI9375_AO8_b; maxdpdcad3_barCAD. NI9375_AO9_b; maxdpdcad4_barCAD. NI9375_AO10_b; maxdpdcad5_barCAD. NI9375_AO11_b; maxdpdcad6_barCAD. NI9375_AO12_b; kappa_comp1_r. NI9375_AO13_b; kappa_comp2_r. NI9375_AO14_b; kappa_comp3_r. NI9375_AO15_b; kappa_comp4_r. NI9375_AI0_b; kappa_comp5_r. NI9375_AI1_b; kappa_comp6_r. NI9375_AI2_b; kappa_exp1_r. NI9375_AI3_b; kappa_exp2_r. NI9375_AI4_b; kappa_exp3_r. NI9375_AI5_b; kappa_exp4_r. NI9375_AI6_b; kappa_exp5_r. NI9375_AI7_b; kappa_exp6_r. NI9375_AI8_b; CylPresWarnings_u64. NI9375_AI9_b; CylPresErrors_u64. NI9375_AI10_b. NI9375_AI11_b. NI9375_AI12_b. NI9375_AI13_b. NI9375_AI14_b. NI9375_AI15_b. |
| ADDITIONAL NOTES: - Variable names are stored in files unique for the project, so that the variable name is automatically propagated to the different chassis. - Names automatically replace ‘-‘ by ‘_’, in order to make the use robust for Matlab - Variables include unit at the end of the name. This is done automatically for the AI channels, and manually when declaring the variable name in the rest of cases. An exception to this naming are variables from MTR PLC. _bar pressure in bar _r ratio (-) _perc percentage (%) _kgs mass flow in kg/s _degC temperature in ºC _b boolean _J energy in Joule _CADATDC crank angle degrees before top dead center _CADBTDC crank angle degrees after top dead center _CAD crank angle degrees _barCAD pressure increase rate bar/CAd _ms time in ms _mA current in mA _V voltage in V - Scripts and functions for loading TDMS in Matlab and Python are provided. - HMI allows to trigger the data logging, and to input test notes. Figure 2. View of the user interface for the file recording. |  |