# APC_9049_RT_main — Open-Loop Commissioning Audit & SIL Plan

Does the cRIO-9049 application (RT + FPGA, APC_9049_RT_main.vi and every sub-VI) work as intended for the first commissioning campaign — engine operation only, open-loop (motoring, then firing with manual spark/DI settings; no closed-loop combustion or plant control)? And how do we exercise it software-in-the-loop, with synthetic data, before anything is fueled or sparked?

Audited 2026-07-08 from the fresh VI exports under original-labview-codebase/ (HTML + full-res diagram GIF/PNG, read page by page), the two FPGA PDFs ([Deck] = 20251020ERD01GUI01 FPGA on cRIO 9049.pdf, [GDoc] = 9049 FPGA VIs - Google Docs.pdf), the Overview PDF, the E1/M1 report ([Report] = 20231130ENG02CWN01_E1_M1report_GUI.docx), and the vendor EPT manual ([EPT-UM] = EPT_VI_UM_RevB.pdf, Drivven/NI Powertrain). Where the old PNG thumbnails and the fresh GIF exports disagree, the GIFs (current live VIs) were used. Caveat: everything here is read from rendered diagrams; items marked ⚠VERIFY need a check in LabVIEW or on the bench — saved front-panel defaults especially, because on a headless RT deployment the compiled panel defaults are the configuration.

Companion tool: tools/gen_cas_traces.py generates synthetic CAS cylinder-pressure cycles (correct 9×7200 shape, phasing, pegging behavior) with a ground-truth truth.json for verifying the analysis chain.

Currency & where to start (as of 2026-07-16). The static audit (F1–F9, §§2–6) stands, with the F3a–F3d dispositions folded in below. Executed since it was written: SIL-0 COMPLETE (§7 — 425 metric comparisons all in tolerance; geometry resolved, picture15; MAPO/IMEPstd columns added); the threshold pipeline is built and — for the first time — connected end-to-end on target (tools/tune_thresholds.py → CylWarningLevels.xml + F3b Load-INI fix); SIL-1 Steps 0–3 done, Step 2 CLOSED (Trig0 follows the sim), and the Step-6 false-trip/latch matrix COMPLETE 7/7 (2026-07-14) via the CAS_loop sim-pressure branch + tools/gen_warning_matrix.py (record sheet docs/cRIO9049 Warning Matrix.xlsx). → Execution order = §7, layered SIL-0 → SIL-1 → SIL-2 → SIL-3, then the §8 pre-fuel checklist. The immediate next action is SIL-1 Steps 4–5 (state-gated spark/DI scheduling + drills) — click-level: docs/sil1-scope-of-work.md §4a–4f and §5a–5h. Pending architecture decision for engine-only running: docs/engine-only-9056-tradeoff.md. Tool pipeline map: tools/README.md.

——————————————————————————————

# 1. The short answer

The 9049 application is in substantially better shape than the 9056 side was: the architecture is sound, the safety anchors are real and fail-safe, and the core engine-synchronous chain (sync → spark/DI scheduling → encoder-clocked acquisition) was already validated at TTL level by the original developer using a physical encoder emulator ([Report], Fig. 20 — an Arduino generating encoder/cam signals, spark & injection TTL outputs loop-verified, CAS processing proven to 4250 rpm and "aligned with NOBLE calculation script").

What was never validated: physical actuation (real coils/injectors), sensor calibration (placeholders in the VI today), the pressure diagnostics added after the 2023 report (they gate spark/DI and, as originally built, false-tripped — see F2/F3; since fixed and matrix-proven 7/7, 2026-07-14: F3b Load-INI root cause + the 6c state gate), and data recording end-to-end.

And the headline for SIL: a virtual crankshaft is built in and vendor-documented. The NI Powertrain EPT position-tracking core on the FPGA has a pattern simulator (EPTControl.SimEnable/SimPeriod, reachable from the TS10ms_loop front panel, saved SimPeriod=500 ≈ 1333 rpm — the developer clearly used it). Per [EPT-UM]: with SimEnable TRUE the physical CrankSig/CamSig inputs are internally disconnected and replaced by simulated counterparts — the simulator generates both the crank and the cam/Z reference for the configured pattern, so full sync is achievable; the feature exists exactly "to bench test the engine control system functionality … without the need for external simulation hardware" (the manual's own FPGA example runs with no external signals wired at all). The exports further show the CAS DAQ sample clock (cRIO_Trig0) is wired from EPT's CrankSigOut, which carries the simulated signal in sim mode ([GDoc]). Flip one boolean and the whole engine-synchronous stack runs against a phantom engine on the real cRIO — no encoder, no rotation. APC_9049_PPhaseCorrection additionally has a built-in canned-pressure mode (UsePcylDatabase). Both are also hazards if left enabled — see the pre-fuel checklist.

# 2. Verdict summary

| VI | Verdict for open-loop commissioning |
| --- | --- |
| RT_main | ✅ Works. Two-frame sequence: SensorCalibration, then CAS/SAVE/TS10ms in parallel forever. The Diagram-Disable frames (Modbus healing loop, legacy System-PID loop) are deleted outright in the live VI (README's "currently Disabled" is outdated). No error gating: loops start even if DAQ-task creation failed — shown NOT benign 2026-07-14: the −200088 DAQmx startup race (known from the 9056, deployed-bringup #5) hit the 9049 after an app restart → CAS loop stuck in a ~1 Hz task-restart cycle, no acquisition until another app restart. Port the 9056 retry-loop fix to `SensorCalibration`. |
| TS10ms_loop | ✅ Works, 3 risks (F1, F5, F6). Gate logic confirmed: enable = ¬CylPressError ∧ ActivateCylinder ∧ UI-enable ∧ (SYSTEMSTATE ≥ 2) per cylinder, spark and DI separately. FPGA WatchdogIn toggled unconditionally each 10 ms tick — fail-safe by dataflow (an upstream error that stops the FPGA write also stops the toggle → FPGA kills outputs). Loop never exits; errors latch to a border indicator only. |
| SparkSettings | ✅ Works. SA/cutoff → CAT conversion correct; dwell hard-coded 4 ms (2–6 clamp) via saved defaults (unwired at call site); cutoff −60 DBTDC from caller. |
| KnockCA50Control | ✅ Inert as configured (both enables come from PC_ControlSettings, off ⇒ manual SA passes through untouched). Not usable for closed loop later without new values (knock retard/advance params saved 0, PID gains dummy, SA clamp ±100 CAD is no protection). |
| ControlSettingsRaster | ⚠ DEFECT-grade echo issues (F4). |
| CAS_loop | ✅ Works. Self-healing acquisition (1 s DAQ timeout → stop/restart task, no error latch-up, ~1 Hz idle retry with no encoder); lossy bounded queues (1000-cycle rolling pre-trigger buffer) — no deadlock/growth path found. Risks: Enque? is an unbound panel control (compiled default decides whether SAVE ever gets data ⚠VERIFY); analytics still run on timeout garbage (feeds F3). |
| SensorCalibration | ❌ NEEDS-CONFIG (F2): pressure "calibrations" are NI-example placeholders. Task: AI voltage, sample clock /cRIO0/cRIO_Trig0 falling ("encoder sample clock"), start trigger cRIO_Trig1 rising, continuous, 7200-buffer. |
| SAVE_loop | ✅ Design sound (flush/buffer/post-mortem paths, no-disk branch), idle-safe. Config risks: Cycles to measure saved 0, BufferingTimeLimit saved 0, DataSaveControl write-back race with the PC UI, panel "No disk" LED saved red ⚠VERIFY recording end-to-end on the bench. |
| 9056SharedVarPolling | ✅ Works; absent/wrong-size 9056 array ⇒ all-NaN ⇒ NaN ≥ 2 = FALSE ⇒ gate fails safe. No staleness check — a frozen 9056 publisher holds the last state forever (F7). |
| PressureAnalytics chain | ⚠ Two structural findings (F2 cal, F3 diagnostics) + known author flags (§6). Data path and per-cylinder re-phasing confirmed correct (offsets 0,4800,2400,6000,1200,3600 ⇔ firing order 1-5-3-6-2-4, consistent with the TS10ms TDC offsets ×10). |
| FPGA_main + EPT/di_supv/esttl | ✅ Safety anchors confirmed (§3). |
| FPGA_IGNDI_supervisor | ✅ Works; indicator-only consumer, but it's a ready-made pass/fail probe for bench spark/DI tests (12 = all channels active). Saved-panel snapshot shows Fault1 = 126 on both NI 9751 modules — decode before trusting DI health telemetry ⚠VERIFY. |

# 3. Safety anchors (confirmed as-built)

FPGA watchdog: EPTControl.WatchdogIn must toggle > 4 Hz; trip shuts

down position tracking and all engine-synchronous outputs (spark/DI die with sync — both consume EPT's FuelSparkSupervisor). RT toggles it every 10 ms unconditionally. Auto-recovery on resumed toggling is undocumented — bench-verify (SIL-1 drill).

Sync-loss latch: MissedCrankFlag/MissedCamFlag latch and block

re-sync until explicitly cleared (UI "Clear Sync Errors" → RT, or the TS10ms panel switch). Confirmed by [EPT-UM] (encoder pattern): Z-edge early / Z-edge missing ⇒ loss of sync, "re-sync is not allowed until the flag is cleared". Falling below stall speed also drops sync (CrankCount/CurrentPosition → 0, SyncStopped TRUE) but does not latch.

Spark Key interlock: spark output (esttl_vt_spark_reva) requires the

Key emitted by the DI supervisor — spark cannot fire unless the NI 9751 DI chain is powered and enabled, even for spark-only tests. Plan bench sessions accordingly (and note: motoring is doubly spark-safe).

Power-up/no-encoder state is safe: modules configured then idle;

unsynced EPT + no DI enables + no Key ⇒ all engine-sync outputs dead until RT connects and sync + state + enables all line up.

RT-side hard gate: spark/DI enables ANDed with SYSTEMSTATE ≥ 2 and

¬CylPressError (per cylinder) — confirmed on the diagram. The state input is the 9056-relayed echo, so the 9056 StateMachine (and its warning clamp + the B0 watchdog) sits above 9049 actuation, as designed.

# 4. Findings that need action (ranked)

F1 — `Override PC settings` bypasses BOTH safety gates. The TS10ms True case substitutes panel-manual enables/settings for the PC-derived ones upstream of nothing: the SYSTEMSTATE ≥ 2 and ¬CylPressError gates are simply not applied in override mode. It's a deliberate dev/bench feature (and genuinely useful for SIL — §7), but it must never ride into a fueled system enabled. ⚠VERIFY saved default = FALSE in LabVIEW; add to the pre-fuel checklist; longer-term consider re-wiring the override downstream of the gates or deleting it from the deployed build.

F2 — Sensor calibrations are placeholders. The CAS task's per-channel custom scales (saved panel defaults of a headless VI) read like NI-example leftovers: slopes 1.00 (one 100.00), intercepts 0/5/10/15/0…, all "bar", ±10 V. Until real transducer calibrations are entered, every pressure number — including the CylPressError decision that gates spark/DI — is meaningless. Enter real cals (and preferably move them to a config file, like the 9056's XML pattern) before any pressure-dependent step. Channel order: [Pcyl1–6, Ppre, Psyst, Pexh] = rows 0–8.

F3 — The pressure diagnostics will false-trip and then silently veto firing. Confirmed semantics: per-category error flags latch (cleared only by the PC CLEAR WARNINGS → APC_MASTER_ClearWarnings → 9049 relay APC_SLAVE_ClearWarnings); any latched error on any cylinder ⇒ 9049_Global_CylPressError = TRUE ⇒ spark and DI gated off. The chain has no state awareness (checks run identically motored or fired), and:

Expected IMEP — the reference for the misfire-from-IMEP check — is an

unwired front-panel control, saved 0.0, not on the connector pane. Trips on any healthy firing and on motoring CORRECTED 2026-07-15 (F3d): the check is one-sided (IMEP ≤ Expected − threshold), so with Expected = 0 it can only fire on IMEP < −threshold — i.e. it is effectively INERT: a silent protection gap (no misfire detection against the intended reference), not a false-tripper. It becomes live — and useful — only once Expected IMEP is actually driven to the true operating value.

CA50 on motored data is noise (the MFB rescale normalizes whatever it gets)

→ CA50max errors latch randomly while motoring.

The first cycle after acquisition start is zero-padded garbage (2-cycle

circular buffer init) → running-IMEP-std / cyclic-variability errors likely latch at every acquisition start.

The deployed thresholds live in an XML on the cRIO (via

LoadWarningConfig → 9049_WarningLevels) whose contents are unknown — possibly never set. ⚠VERIFY on target.

Confirmed from the exports (2026-07-11 — `LoadWarningConfig` / `Pcyl_Diag` / `CombCluster2Array`). Pcyl_Diag compares each metric to a Warning and an Error limit and emits two flag clusters; CombCluster2Array treats them differently — this is the crucial detail:

Warnings (index 0) → simple OR → the WarningsAndErrors indicator only

(advisory, non-latching).

Errors (index 1) → feedback-node latch per category → OR across all →

9049_Global_CylPressError. Only the Error level latches and vetoes fire; the latch clears only when APC_SLAVE_ClearWarnings fires (True case).

The exact field↔flag↔metric map (each field has a …Warning and …Error variant; the trip is metric > threshold):

| Error flag | 9049_WarningLevels field | metric compared |  |  |
| --- | --- | --- | --- | --- |
| maximum pressure | MaxPCylMax | Pmax |  |  |
| cyclic variability | MaxIMEPstd | running IMEPn std |  |  |
| misfire cyl-to-cyl | MaxDevFromAvg | \ | IMEPg − cyl-mean\ |  |
| misfire self reg | MaxDevFromSelfAvg | \ | IMEPn − own running mean\ |  |
| misfire from IMEP | MaxDevFromExpectedIMEP | \ | IMEPn − Expected IMEP\ |  |
| knock | MAPOmax | MAPO |  |  |
| late combustion | CA50max | CA50 |  |  |

plus samples for running IMEP std (I32). tools/tune_thresholds.py emits this exact set (Warning+Error) from a SIL metrics CSV.

F3a — disarm-by-threshold FAILS for the phasing checks on no-combustion data (confirmed live 2026-07-14, SimEnable). With CA50maxError set to the disarm sentinel 1e6 (verified loaded in 9049_WarningLevels via the UI), the late-combustion error still latches in SimEnable mode. Root cause: with no real combustion the pressure is flat (floating 9222 / sim), so the MFB normalization inside APC_HRL→APC_9049_HRL_apparentHRL divides by HRmax−HRmin ≈ 0 → CA50 (and CA03/CA10/CA90/CA97) come out non-finite (NaN/Inf) — which beats any finite limit (+Inf ≥ 1e6 is TRUE; a NaN defeats a negated/in-range compare). The -99 sentinel array in APC_HRL shows the author anticipated CA-detection failure, but that default is not reaching the comparison in this mode. Implications: (1) you cannot "disarm" a phasing check by raising its threshold — the diagnostics need a validity guard / non-finite coerce (a combustion detected? gate that skips CA-based checks when HRmax−HRmin ≈ 0) before the rebuild; (2) for SIL-1 the phasing checks can only be exercised with a real pressure source driving the CAS channels (all 6 Pcyl, since late-combustion is per-cylinder OR'd), not turned off by a big number. Confirm the returned value with trace-sets/flat_set/ through the SIL-0 harness, or by probing the live CA50 on the panel. Same family as the NaN-at-Unflatten bench bug — non-finite sanitization is a systemic gap in the analytics (F8). Live confirmation 2026-07-14 (SIL-1 sim-pressure, clean motored stream): late-combustion errors latch continuously on random cylinders (different subset each run) despite CA50max = 1e6, on trace data whose every other metric is clean (IMEP σ 0.06, Pmax σ 0.06). The CA50 trend shows the mechanism: motored CA50 noise band −25…−32 punctured by constant spikes pinned at ≈ +45 (the detector's search-window end) — the crossing detector fails on no-combustion data, usually clamping to the window end and intermittently returning non-finite, which beats any finite threshold. This is F3's "CA50max errors latch randomly while motoring", demonstrated with the disarm in place. The max-pressure warning row re-tripping at Pmax≈151 vs limit 181 shows garbage values transiently reach the other comparisons too. Operational remedies: (1) CLEAR after acquisition starts; (2) baseline-motored acceptance = the other six flags clean (late-combustion latches are known noise); (3) for spark/DI gate tests stream a fired set — fired CA50 is stable (~12.9), so CylPressError stays FALSE. Structural remedy — BUILT and live-verified both directions (2026-07-14): the state gate — late-combustion AND misfire-from-IMEP warning+error flags AND'd with 9049_Global_SYSTEMSTATE ≥ 2 in CombCluster2Array, upstream of the error latch (late combustion is only meaningful when combustion is commanded; same signal already gates spark/DI, so stale/NaN state fails consistent-safe). This is the F3 "no state awareness" root fix and also silences the finite noise clamps; the matrix late_combustion set proved the re-arm direction (ERROR ×6 at state ≥ 2). As-built record + verification evidence: `docs/sil1-scope-of-work.md` Step 6c. (Non-finite CA sanitization to −99 in APC_HRL stays as backlog hygiene for telemetry + the future CA50 closed loop — pattern recorded in 6c's closing note.)

Actions: build a motoring threshold set (MaxDevFromExpectedIMEP, CA50max and MAPOmax disarmed — motoring can't knock and has no combustion phasing; IMEP-std and dev-from-avg generous; MaxPCylMax ≈ observed motored peak ×1.3, capped at the hardware ceiling) and a separate fired set. tools/tune_thresholds.py emits both directly from a metrics CSV. Knock (`MAPOmax`) and cyclic-variability (`MaxIMEPstd`) auto-tune only if the harness CSV carries `mapo`/`imepstd` columns — add them to the APC_SIL0_HRL_Desktop Build-Array (both are already in CombustionAnalysisCluster); until then MAPOmax stays hand-set for firing. Add the procedural step "after sync is stable under motoring: CLEAR WARNINGS, confirm `CylPressError` = FALSE, then request IDLING/FIRING". The Expected-IMEP fix is DECIDED (F3d, 2026-07-15): wire it from the commanded `IMEP-REF` — as-built it is not a false-tripper but INERT dead weight (one-sided check vs 0), a silent protection gap until wired.

F3b — the warning thresholds in the comparison path are DISCONNECTED from the FGV/XML (root cause found by live probe, 2026-07-14). Probes in Pcyl_Diag's diagnose path: data wire = Pmax ✓ (150.83–150.95), but MaxPCylMaxWarning = 100 and MaxPCylMaxError = 1000 — while the 9049_WarningLevels FGV (and the UI Retrieve display) holds the loaded XML values 213.5/231.3. The comparisons consume Pcyl_Diag's local threshold values (saved panel defaults), which the LoadINI/Set management cases never effectively update — so every threshold entered via UI/XML reaches the display but NOT the protections. Consequences: the eternal all-six max-pressure warning (Pmax 151/178 > 100); and F3a is likely re-explained — "late-combustion trips despite CA50max=1e6" = finite +45 noise spikes beating the default (~15), no non-finite values required (re-test the 1e6 disarm after reconnection; non-finite sanitization stays as hygiene). ROOT CAUSE FOUND AND FIXED (2026-07-14): Pcyl_Diag's `Load INI on startup` control was saved FALSE — the running 9049 never loaded any warning XML; the comparisons ran on panel defaults (MaxPCylMax 100/1000, CA50max ≈15, …) since the system was built, while the display pipeline (XML → FGV → UI Retrieve) showed the loaded values. Scope had been confirmed by probes (CA50maxError ≈15 vs loaded 1e6 — all fields; VI non-reentrant). User set the control TRUE and saved it as default. This is F6 (compiled-default configuration debt) at its most consequential — and it re-explains F3a: the "1e6-disarm-beating" late-combustion trips were finite +45 CA50 noise beating the default 15. Residual verifications (docs/sil1-scope-of-work.md Step 6d): motored → max-pressure row green; re-test the CA50 1e6 disarm; check whether UI RELOAD INI/SET reach the compares mid-run (else XML swaps require an app restart); carry Load INI on startup = TRUE into every deployed build (§8 checklist). Commissioning lesson: verify protections by probe/behavior, never by the UI display — the display reads the FGV, not the comparisons.

F3c — flag-order mismatch between `Pcyl_Diag` and the UI convention (found 2026-07-14, user). Pcyl_Diag's Warnings/Errors cluster order is [maxP, cyclic variability, cyl-to-cyl, self reg, IMEPref, knock, late] (cyclic variability 2nd) while CombCluster2Array's unbundles and the _UI_Errors label table use [maxP, self reg, cyl-to-cyl, IMEPref, cyclic variability, MAPO, late] (5th). Name-bound nodes are immune; any POSITIONAL consumer (U64 byte packing, Warning2DColorMap reshape, a Cluster-to-Array anywhere) scrambles metrics 2–5 between display and label — a latent mislabeling hazard in the safety display. RESOLVED as no active mislabeling (2026-07-14): the single-flag injection (`variability` set) tripped cyclic variability at display row 5 — grid matches labels. The typedef-order mismatch remains as latent debt: align the orders in the rebuild so no future positional consumer can scramble. Debugging pattern worth keeping: CylPresWarnings/CylPresErrors decode as one byte per metric (first metric in the top byte), bit-per-cylinder — a complete remote probe of the flag state.

F3d — all three misfire deviations are ONE-SIDED (low-side) checks (found 2026-07-15, user, from the `Pcyl_Diag` print). Each is built as IMEP ≤ (reference − threshold) with no absolute value: misfire-from-IMEP (reference = Expected IMEP), misfire cyl-to-cyl (fleet mean), misfire self reg (own running mean). Confirmed by the matrix drill: in the misfire set only the LOW cylinder tripped cyl-to-cyl (the five above the dragged-down mean never can). Defensible as design intent — a misfire only removes work; the high side is covered by Pmax/MAPO — but three consequences: (1) the MaxDevFrom… field names mislead (they read as |deviation|); (2) with the unwired Expected = 0, misfire-from-IMEP is INERT (see the corrected F3 bullet) — arm it by driving Expected IMEP when firing; (3) over-performance (over-fueling, IMEP high) is deliberately NOT caught by these checks — verify Pmax/knock margins carry that duty in the commissioning book. DECISION (2026-07-15): do NOT add Abs / two-sided compares to these checks. Rationale: misfire is inherently low-side; these flags latch a whole-engine spark/DI veto, and a two-sided MaxDevFromSelfAvg would false- veto on every load/speed-increase transient (IMEP rising above its own running mean) — re-creating F3-class false trips; the high side belongs to Pmax/MAPO. If high-side IMEP detection is ever wanted, build it in the rebuild as a separate warning-level (non-latching) check with its own thresholds — misfire drops are ~27 bar, over-performance excess ~1–3 bar; one shared limit cannot serve both sides. The actionable F3d item is arming, not symmetry: wire `Expected IMEP` from the commanded value (`IMEP-REF` in the PC command cluster) so misfire-from-IMEP goes live when firing. Any future Pcyl_Diag change ⇒ re-run the false-trip matrix (tools/gen_warning_matrix.py + SOW Step 6b) as the regression gate. Naming/typedef cleanup belongs with the F3c order alignment in the rebuild.

F4 — `9049_ControlSettings` echo is not what consumers think. Fresh export shows: element [0] is wired to the `PFI0 mode` panel control (constant 0), not the system state (author note: "…used for broadcasting current combustion mode through PFI0"); [1] InjectionEnable and [5] SparkEnable pass through Boolean-Array-To-Number — i.e. 6-cylinder bitmasks 0–63, not 1/0 (this contradicts the earlier 2026-07-07 reading recorded in supervisory/monarch/settings_9049.py; a live capture decides — drive all six cylinders enabled and read whether [1] is 1 or 63); and the raster taps the pre-Override wires, so in override mode the echo reports what the PC would have commanded, not what fires. Consequences: the PC/9056/Python consumers of the echo see a stale state 0 in [0] today. Reconcile with a live capture during SIL-1, then fix settings_9049.py to match reality.

F5 — Simulation provisions must be OFF for any fueled run. Saved panel values found: SimPeriod = 500 (and a non-zero EPTData snapshot — the sim was used); UsePcylDatabase render ambiguous. A build deployed with SimEnable TRUE "syncs" happily to a phantom engine; with UsePcylDatabase TRUE the diagnostics run on canned data. Both ⚠VERIFY = FALSE before deployment; both belong on the pre-fuel checklist permanently.

F6 — Compiled-default configuration debt (headless deploy). Several behavior-defining controls are unbound panel controls whose saved value is the deployed configuration: Override PC settings, Enque? (CAS→SAVE data flow!), 1/2 Z pulse, Must Use Cam & Z, toggle TDC, the DIControl enable switches, SimEnable/SimPeriod, Cycles to measure, BufferingTimeLimit. ⚠VERIFY each in LabVIEW and record the intended values in a deployment sheet (candidate for a small INI, same pattern as the 9056).

F7 — No 9049-side staleness guard on the 9056 state relay. If the 9056 stops publishing but the value stays (process frozen), the 9049 holds the last SYSTEMSTATE indefinitely — including ≥ 2, which keeps spark/DI enabled with a dead supervisor above them. (Total absence is safe: NaN → gate closed. And the 9056-side WatchDog covers the reverse direction, 9049-loss → SAFE.) Cheap mitigation when wanted: stall-count TimeStamp9056 (element 0 of the polled array) in CAS_loop and treat stale as state −1. Log as a commissioning watch item; same family as the closed loss-of-PC gap.

F8 — Analytics accuracy debt (author-flagged, inherited). HRL_volume: "FIX STROKE CALC", crankpin-offset inaccuracy, IVC/EVO/EVC "to be updated" ⇒ absolute IMEP bias (motored and fired); threshold calibration inherits it. APC_9049_CA: the CA97 detector constant renders as 0.9 (same as CA90) — if real, CA97/CA10CA90 are wrong ⚠VERIFY. Pegging window = cylinder-frame samples 6900–7100 (+330…+350 CADATDC), additive peg to LP-filtered exhaust — fine, but Pmax-class metrics are offset-sensitive to a bad exhaust channel. None of this blocks motoring; it blocks trusting absolute numbers. The 2023 "aligned with NOBLE calculation script" claim gives the cross-check route: rerun that alignment via SIL-0 with truth.json.

F9 — DI hardware unknowns for the firing step (from [Report], still open). Injector current profile inherited from a single-cylinder project ("could be biased"); physical actuation of all six never verified; 9751 Fault1 = 126 snapshot to decode. Bench DI dry-fire (SIL-3) before fuel.

# 5. What the open-loop use case does / doesn't need

Not needed (and confirmed inert when off): KnockCA50 closed loop (both enables off ⇒ manual SA passthrough), PFI (strategy replaced by intake flow regulator), MTR/membrane, all 9056 closed-loop combustion-adjacent modes (9056 loops run mode 0/1), Modbus healing loop + legacy System-PID loop (deleted from RT_main). The dyno speed loop and thermal loops are 9056 territory and out of scope here.

Needed and used: EPT sync chain, spark path (manual SA + fixed 4 ms dwell, cutoff −60), DI path (manual duration/SOI, window constants now 200/0 DBTDC on the diagram — matches [GDoc]; the older 90/−30 reading is outdated), per-cylinder activate flags, CAS acquisition + analytics (as monitor + CylPressError gate), SAVE recording (the commissioning product), the state echo, and both watchdog directions.

Minimal command surface (all already modeled in supervisory/monarch/control_settings.py): Activate cylinder ×6, DI enable + duration + advance, IGN enable + spark advance, CA50/Knock control = OFF, requested mode, speed selector. The existing Python command path (Phase B/C) can drive all of it.

# 6. On "working as intended" — the honest scoreboard

Proven by the 2023 campaign ([Report]): sync + filtering + TDC toggle;

CAS clock/trigger generation; 6-cyl CAS processing to 4250 rpm; spark/DI TTL outputs incl. per-cylinder disconnect; TDMS logging toolchain.

Confirmed by this audit (static): gate logic, watchdog dataflow,

restart/queue robustness, phasing consistency, safety anchors, limiter independence (9056 above 9049).

Broken/misleading as-built: F1–F4 above.

Unprovable without a bench run: watchdog auto-recovery, deployed panel

defaults, warning-XML contents, Trig0-follows-sim, recording end-to-end, echo element semantics — exactly the SIL-1 agenda below.

# 7. SIL plan — layered, cheapest first

SIL-0 — analytics on the dev PC (no hardware). — MATH VALIDATED (2026-07-11). → Click-level step-by-step: `docs/sil0-scope-of-work.md`. Generate traces (real MONARCH geometry, from picture15 — bore 0.112, ½-stroke Lm=0.0745, rod Lb=0.217, CR 12.8, offset −0.00099): python tools/gen_cas_traces.py trace-sets/sil0_traces --cycles 20 --mode mixed --fire-from 10 --misfire 3:14 --knock 1:16:2.5 --bore 0.112 --stroke 0.149 --conrod 0.217 --cr 12.8 --pin-offset=-0.00099 [--q-fired 6000]. The generator emits `cycle_NNNN_phased.csv` (6×7200, already phased, TDC at sample 3600) + truth.json.

Harness built: APC_SIL0_HRL_Desktop.vi on My Computer (globals-free — sidesteps the 9049_Global_CylPressError target-relative trap): List Folder (*_phased.csv) → For loop → Read Delimited Spreadsheet (comma delimiter) → `support/APC_HRL.vi` with Pcyl=6×7200, exhaust pressure=flat Initialize Array(3.0, 7200), FilterCoefs from HRL_CreateFilter (Expected IMEP left unwired — feeds only the misfire diag). APC_HRL has no geometry input (internal VOL constant) and runs its own For loop over the 6 cylinders, so its cluster fields (IMEPg/IMEPn/Pmax/CA50) come out as 6-element arrays. Inner loop over those → rows [cycle, cyl, IMEPg, IMEPn, Pmax, CA50] → concatenating tunnel → Write Delimited Spreadsheet → labview_metrics.csv. Score with tools/compare_hrl.py truth.json labview_metrics.csv.

Result (2026-07-11): 425 comparisons, ALL within tolerance — worst |Δ| IMEPg 0.001 bar, IMEPn 0.000, Pmax 0.002 bar, CA50 0.112 CADATDC, across all 20 cycles × 6 cylinders (motored + fired). The 9049 HRL/IMEP/CA50 math is faithful vs known truth (F8 verdict: the math is sound; any absolute IMEP bias would show as a consistent offset — none seen at this geometry). Python side (generator + `compare_hrl.py` + tests) done and self-tested.

~~Still to do in SIL-0~~ — ✅ COMPLETE (thresholds 2026-07-13/14; false-trip matrix 7/7 in SIL-1 Step 6, 2026-07-14 — `tools/gen_warning_matrix.py`, record sheet `docs/cRIO9049 Warning Matrix.xlsx`). The steps below are kept as the repeatable recipe (they are the regression procedure whenever Pcyl_Diag or the thresholds change — F3d rule). This produces the two warning-XML profiles (motoring, fired) the 9049 loads at run time, and proves the false-trip/latch behaviour F3 warns about. Scope boundary: the desktop harness runs support/APC_HRL.vi only (it computes the metrics), so Steps 1–4 (generate → metrics → thresholds) run fully off-hardware; the latch tests (Step 5) need the Pcyl_Diag → CombCluster2Array → 9049_Global_CylPressError chain and are best run in SIL-1 on the real 9049 — that global is a target-relative single-process variable (the trap that kept those VIs off the desktop harness).

Step 1 — Generate the motored trace set.

python tools/gen_cas_traces.py trace-sets/motored_set --cycles 30 --mode motored \

--bore 0.112 --stroke 0.149 --conrod 0.217 --cr 12.8 --pin-offset=-0.00099

Writes into trace-sets/motored_set/: raw cycle_NNNN.csv (9×7200 — Pcyl1–6, Ppre, Psyst, Pexh), `cycle_NNNN_phased.csv` (6×7200, each cylinder already in its own frame, TDC at sample 3600 — these feed `APC_HRL` directly), and truth.json (the computed IMEPg/IMEPn/Pmax/CA50 per cycle·cylinder). --mode motored = compression/expansion only, no combustion, so IMEP ≈ −1 bar and Pmax is the motored peak (~150 bar at this geometry/CR). Geometry is the picture15 as-built (--stroke 0.149 = 2×half-stroke 0.0745); raise --cycles for tighter statistics. Accept: 30 *_phased.csv + truth.json present; the generator prints the per-cycle period.

Step 2 — Run the set through the harness → metrics, and re-confirm the math. Point APC_SIL0_HRL_Desktop.vi (My Computer) at trace-sets/motored_set/: List Folder (*_phased.csv) → For loop → Read Delimited Spreadsheet (comma delimiter) → support/APC_HRL.vi with Pcyl = the 6×7200 array, exhaust pressure = Initialize Array(3.0, 7200) (flat — must be wired or pegging → NaN), FilterCoefs from HRL_CreateFilter. APC_HRL runs its own loop over the 6 cylinders, so its IMEPg/IMEPn/Pmax/CA50 come out as 6-element arrays; the inner loop writes one row per (cycle, cyl) — [cycle, cyl, IMEPg, IMEPn, Pmax, CA50], cycle = outer i+1, cyl = inner i+1 — through a concatenating tunnel → Write Delimited Spreadsheet → trace-sets/motored_set/labview_metrics.csv. Then sanity-check before trusting any threshold derived from it:

python tools/compare_hrl.py trace-sets/motored_set/truth.json trace-sets/motored_set/labview_metrics.csv

Accept: labview_metrics.csv has 30×6 = 180 numeric rows and compare_hrl prints "ALL metrics within tolerance". (If it fails, fix the harness before Step 3 — every threshold below is derived from these numbers.)

Step 3 — Recommend and enter the motoring thresholds.

python tools/tune_thresholds.py trace-sets/motored_set/labview_metrics.csv --mode motored \

--pmax-hard-limit <engine over-pressure ceiling, bar>

Prints the full 9049_WarningLevels set (each field as <field>Warning / <field>Error) plus the samples for running IMEP std window, with rationale:

`MaxPCylMax` — Warning ≈ peak×1.2, Error ≈ min(peak×1.3, --pmax-hard-limit).

The ceiling is the engine's physical over-pressure limit (get the number from the team); the Error limit is never allowed above it.

`MaxIMEPstd` / `MaxDevFromAvg` / `MaxDevFromSelfAvg` — generous (motored

cycles are repeatable; these mainly guard against acquisition glitches).

DISARMED for motoring (the XML field set to the huge sentinel — effectively

off): `CA50max` (motored CA50 is MFB noise), `MaxDevFromExpectedIMEP` (the unwired-Expected-IMEP-0 trap, F3), `MAPOmax` (motoring can't knock).

Enter each Warning + Error into the UI CYLINDER (Errors) screen (APC_PC_UI_Errors.vi) → this sets 9049_WarningLevels (SetWarningLimits); Save to INI (SaveWarningLimitsToINI) so it survives redeploy, and LoadWarningConfig loads it into the running 9049 at startup. Adjust margins with --pmax-margin (default 1.30), --std-margin (5.0), --imep-std-samples (20 = the I32 std window). Accept: the motoring profile saved on the 9049; a CLEAR WARNINGS then CylPressError = FALSE confirmed before the first IDLING request.

Keep a separate fired profile. Once fired cycles exist (a --mode fired synthetic set now, or captured light-off data later): same harness → tune_thresholds … --mode fired --pmax-hard-limit <ceiling>. Fired mode arms CA50max and MaxDevFromExpectedIMEP from the data — but the latter still requires Expected IMEP actually driven (else leave it disarmed); MAPOmax stays hand-set unless you add the MAPO column (Step 4). Save it as a second XML profile; never fire on the motoring set.

⚠ The two-profile scheme is NOT built — this is new work, not "save a second file."

The as-built 9049 has a single profile slot: one 9049_WarningLevels variable

persisted to one hardcoded CylWarningLevels.xml (APC_9049_LoadWarningConfig.vi,

no state input), and the UI edits that one slot — SaveWarningLimitsToINI overwrites

the file, it does not keep two. A real motoring/fired split needs: (a) two persisted

profiles; (b) a state-driven selector on the 9049 that swaps the active limits at the

MOTORING↔FIRING transition; (c) UI to edit each profile; (d) fail-safe handling —

load-fail ⇒ hold the tighter set or safe-hold, tie the swap to actual (not commanded)

state, and manage the CylPressError latch so the swap itself doesn't nuisance-trip.

Until it's built, treat the two profiles as operator-swapped by hand (re-enter + Save

the correct set before changing run type). Tracked in docs/migration-seam.md (backlog,

item 3).

Step 4 — (optional) add `MAPO` + `IMEPstd` harness columns to tune knock + cyclic-variability from data. MAPOmax (knock) and MaxIMEPstd (cyclic variability) can't be derived from IMEP/Pmax/CA50 — they need the analytics' own MAPO and IMEPstd, which APC_HRL already computes inside CombustionAnalysisCluster; the harness just isn't writing them out. In APC_SIL0_HRL_Desktop.vi, unbundle `MAPO [bar/CAD]` and `IMEPstd [bar]` alongside the existing four, extend the inner-loop Build Array, and write a header row naming the columns (the tool matches by name — mapo, imepstd, order-independent):

cycle,cylinder,imep_g,imep_n,pmax,mapo,ca50,imepstd

Re-run tune_thresholds: the report now reads "analytics IMEPstd column" and, in --mode fired, emits a numeric MAPOmax (×2/×3 of observed clean-combustion MAPO — a starting point to refine from a knock-onset sweep). Motored still disarms MAPOmax (no knock) but tightens MaxIMEPstd from the real column.

Step 5 — the false-trip / latch matrix + clear-warnings path (best done in SIL-1 — needs the diagnostic chain, not just APC_HRL). Purpose: prove a genuine fault latches 9049_Global_CylPressError (which vetoes spark/DI) and that CLEAR WARNINGS releases it — the behaviour F3 warns about. TOOLED (2026-07-14): `python tools/gen_warning_matrix.py` emits the whole drill — 7 trace sets (clean controls + one per fault), an all-armed CylWarningLevels.drill.xml, and a manifest with the computed expected Warning/Error per set; run it via the CAS_loop sim-pressure branch per docs/sil1-scope-of-work.md Step 6. The underlying fault recipes:

Misfire — --misfire <cyl>:<cycle>: a dead cylinder → IMEP collapses →

misfire from IMEP / misfire self reg / misfire cyl-to-cyl Errors → CylPressError latches.

Knock — --knock <cyl>:<cycle>:<intensity>: pressure oscillation → MAPO

spikes → knock Error (requires MAPOmax armed + the MAPO column, Step 4).

First-cycle garbage — the 9049's 2-cycle circular buffer makes the first

acquired cycle zero-padded → cyclic variability may latch at every acquisition start; confirm the MaxIMEPstd floor (or a skip-first-cycle) suppresses it.

Then exercise the clear path: PC CLEAR WARNINGS → `APC_MASTER_ClearWarnings` → 9049 relay `APC_SLAVE_ClearWarnings` resets the feedback-node latches → CylPressError returns FALSE and spark/DI gating is restored. SIL-0 output: the finalized motoring + fired warning-XML profiles, plus a documented false-trip matrix (which fault trips which metric at the chosen limits) in the commissioning book.

SIL-1 — virtual crankshaft on the real 9049 (EPT simulator). → Click-level step-by-step: `docs/sil1-scope-of-work.md`. Run RT_main interactively from the dev environment (or a bench build), HV/fuel physically absent. SimEnable TRUE; SimPeriod = 60·4e7/(rpm·3600) → 741 ≈ 900 rpm, 370 ≈ 1800 rpm (or use the vendor speed2ticks.vi; the simulated signals are also mirrored on SimCrankSig/SimCamSig indicators for scope checks [EPT-UM]). Note the simulator injects at the EPT input, so the ½-Z synthetic-cam logic, deglitch filters, and NI 9401 wiring are NOT exercised — that's SIL-2's job. Agenda:

Sync acquired (CrankStalled/SyncStopped clear, CrankCount/CurrentPosition

rolling, Speed(RPM) correct).

Does Trig0 follow the sim? The FPGA export shows cRIO_Trig0 wired

from CrankSigOut (which carries simulated signals in sim mode), so CAS acquisition should clock — the Deck's "Trig0 = ENC A" text describes pass-through. Confirm: CAS_loop stops timing out and delivers 7200-sample cycles (of whatever is on the 9222 terminals). → CONFIRMED on the bench (2026-07-14): `rpm from DAQ` = the TS10ms-set rpm, so Trig0 clocks acquisition in sim mode.

Spark/DI scheduling: enable via PC_ControlSettings with the state gate

satisfied (drive 9049_Global_SYSTEMSTATE ≥ 2 by running the 9056, or SV injection, or — bench-only — Override mode w/ F1 noted); observe NumberOfActiveIGN_DI = 12, scope the Mod4 spark and Mod5/6 DI outputs, sweep SA/SOI from the Python command path and watch dT inj/SparkOut.

Drills: watchdog (stop the RT loop → outputs die; resume → document

recovery behavior), sync-loss inject + clear, state-gate walk (spark/DI dead below state 2), CylPressError veto + clear, echo live-capture (F4), recording drill (REC → TDMS files appear, Enque? answered). Pressure inputs during SIL-1 are whatever the 9222 terminals float at — use the SIL-0-derived open thresholds, or drive 1–2 channels from a function generator/AO, or (optional) the Seam-A sim-read below.

SIL-2 — physical encoder emulation (the 2023 rig, resurrected). Feed real A(+B)+Z(+cam) TTL into Mod3 DIO0–3: the Arduino emulator from [Report] Fig. 20 if it still exists in the lab, else a ~$20 microcontroller (in 1/2 Z pulse cam-less mode only A + Z are required; 1800 rpm = 108 kHz A, 30 Hz Z, phase-locked — beyond typical two-channel function generators). Adds coverage the internal sim can't: NI 9401 input path, deglitch filters, the synthetic-cam logic, real quadrature. Also the path to speed-transient tests the internal sim can't do without RT-side SimPeriod ramping.

SIL-3 — actuation dry tests (commissioning entry, after SIL-1/2 pass). Spark coils on bench plugs; DI drivers into dummy loads/disconnected injectors; remember the Key interlock (9751s must be powered+enabled for any spark). Decode Fault1=126 first (F9). This is the boundary where the team's commissioning plan takes over.

Optional Seam-A/B build (full-chain SIL with no cRIO at all): CAS_loop's DAQmx Read wrapped in a sim case that reads cycle_????.csv (NOT cycle_*.csv — that also matches the _phased files and poisons the metrics) + waits one cycle period (133 ms @ 900 rpm — printed by the generator); SensorCalibration's task-creation frame already sits in a Diagram-Disable whose empty frame is the natural stub. (The Seam-A sim case is now BUILT in CAS_loop and ran the whole Step-6 matrix — `docs/sil1-scope-of-work.md` 6a; this note remains only for the no-cRIO desktop variant.) Worth building if the cRIO bench is a bottleneck; otherwise SIL-1 covers more with less LabVIEW surgery.

# 8. Pre-fuel checklist (add to the commissioning book)

Every item ⚠ must be re-verified on the deployed build, not the dev copy:

Override PC settings = FALSE (F1) — and nobody's bench build leaks in.

SimEnable = FALSE, UsePcylDatabase = FALSE, `SIM pressure?` = FALSE

(the CAS_loop sim-read branch, if built — sil1-scope-of-work.md Step 6a) (F5).

Real sensor calibrations entered + spot-checked (F2).

Motoring warning XML loaded — not the all-armed `CylWarningLevels.drill.xml`

(drill-sized, uncapped Pmax); CLEAR WARNINGS drill done; CylPressError = FALSE confirmed before first IDLING request (F3). 4b. `Load INI on startup` = TRUE on the deployed build (F3b — saved FALSE meant the XML was NEVER loaded and the comparisons ran on panel defaults while the UI displayed the loaded values). Verify the ACTIVE thresholds by probe or trip-behavior, never by the UI display (the display reads the FGV, not the comparisons).

Enque? = TRUE + a recording drill has produced readable TDMS (F6).

Encoder mode flags (1/2 Z pulse, Must Use Cam & Z, toggle TDC)

match the actual engine sensor set.

Watchdog drill + sync-loss drill passed on the deployed build (SIL-1 §4).

State-gate drill: spark/DI provably dead below IDLING from the real

command path (not Override).

9049_ControlSettings echo reconciled with a live capture; Python

settings_9049.py updated (F4).

E-stop path exercised end-to-end with the 9049 in the loop.

# 9. Open questions

Trig0-in-sim (decides SIL-1 item 2) RESOLVED (2026-07-14, bench): YES. With the EPT

sim running, APC_9049_CAS_loop's rpm from DAQ tracked the SimPeriod rpm set on TS10ms — so cRIO_Trig0 (from CrankSigOut) clocks CAS acquisition in sim mode. Acquisition testing does not need SIL-2's real encoder. (Also confirmed same session: Graph time delivers full 7200-sample cycles + DAQ error 0 — Step 2 fully closed.)

Watchdog auto-recovery semantics (SIL-1 drill).

Deployed warning-XML values; Enque?/Override/Sim saved defaults.

Echo bitmask vs 1/0 (F4) — live capture.

Does the Arduino encoder emulator still exist in the lab? (SIL-2)

Fault1 = 126 meaning (NI 9751 manual) (F9).

Engine geometry values RESOLVED — bore 0.112 / stroke 0.149 / rod

0.217 / CR 12.8 / pin-offset −0.00099 (picture15), used in SIL-0 and validated (§7, 425 comparisons in tolerance). Remaining: confirm picture15 matches the as-built engine, and the working-fluid (argon) kappas for the analytics.